Microwave Setup Parts:

- Microwave Generator (Rhode & Schwartz: SMT 06)

- 2-4 GHz Isolator (Pasternack: PE8301)

- 3-8 GHz Frequency Doubler (Marki Microwave: D-0308)

- Bandpass Filter 5.6-7.0 GHz (Mini-Circuits: VBFZ-6260)

- 2-8 GHz Microwave Switch (Pulsar Microwave: SW1AD-15)

- 5.9-6.4 GHz Amplifier (Microwave Power: L0607-37)

- 4-8 GHz Isolator (Pasternack: PE8302)

- 30dB Directional Coupler (Pasternack: PE2204-30)

- 4-8 GHz Circulator (Pasternack: PE8402)

- Power Meter (Mini-Circuits: ZX47-40+)

- Alternate Power Meter- passive (Pasternack: PE8016)

- Attenuators (5W, 10dB; 2W 10dB)

- Stub-tuner (Maury Microwave: 1719C)

- Waveguide (Precision Waveguide: CPR137 Flanges, n-type connector  6” open-ended waveguide)

Elements 5, 6 and 10 are active and require power supplies to drive them.  (+/- 5VDC, low current for #5 and #10 and +15VDC, 1-5A for #6)